1.  Manufactured and certified by  	_______  Enerflex  Ltd., 3615 34th Street N.E.  Calgary, Alberta, T1Y 6Z8 Canada________________________________________

(Name and address of Manufacturer)

2.  Manufactured for  	Sterling Energy Comp. 1225 17th st, Suite 2520. Denver Colorado, CO 80202. USA

(Name and address of Purchaser)

3.  Location of installation  	Yenter Gas Processing Plant. Sterling Colorado. USA

(Name and address)

4.  Type  	V-1 SUCTION SCRUBBER, VERT.	   	10569B211	   	P1640.2	    	 10570-401 REV.1                 	623	   	2013

(Horiz. or vert. tank)	(Mfgr's serial No.)	(CRN)	  (Drawing no.)	(Nat'l Bd. No.)	(Year built)

5. The chemical and physical properties of all parts meet the requirements of material specifications of the ASME BOILER AND PRESSURE                                   VESSEL CODE.  The design, construction, and workmanship conform to ASME Rules, Section VIII, Division 1  	2010

Year

to  	2011	   	-	   	--

Addenda (Date)	Code Case Nos.	Special Service per UG 120(d)

6.  Shell:  		SA-106-B	   	0.500"	   	0.0625"	   	0', 11.75"	   	5', 0" S/S

Matl. (Spec. No., Grade)	Nom. Thk. (in.)	Corr. Allow. (in.)	Diam. I.D. (ft. & in.)	Length (overall) (ft. & in.)

7.  Seams:		SMLS			NONE			100			-		    - 			TYPE 1			FULL			100			    1______

Long. (Welded, Dbl.,	          R.T. (Spot                Eff. (%)                  H.T.                  	Time (hr)           Girth (Welded, Dbl.,             R.T. (Spot,	              Eff           No. of Courses

Sngl., Lap, Butt)                or Full )                                          Temp (ºF)	                                         Sngl., Lap, Butt)                 or Full)                    (%)

8.  Heads:  (a)  Matl.  	                  SA-234-WPB	   (b) Matl.  	SA-234-WPB

(Spec No., Grade)	(Spec No., Grade)

| (a) | Location (Top, Bottom, Ends) | Minimum Thickness | Corrosion Allowance | Crown Radius | Knuckle Radius | Elliptical Ratio | Conical Apex Angle | Hemispherical Radius | Flat Diameter | Side to Pressure (Convex or Concave) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TOP | 0.500" | 0.0625” ” | -- | -- | WELD CAP | -- | -- | -- | CONCAVE |
|  | BOTTOM | 0.500" | 0.0625” | -- | -- | WELD CAP | -- | -- | -- | CONCAVE |

If removable, bolts used (describe other fastenings)  	---

(b)

(Matl., Spec. No., Gr., Size, No.)

9.  MAWP                   655                     	                                        N/A                             psi at max. temp.                             300                                                     N/A              	      ºF

(internal) 	               (external) 	           (internal)	             (external)

Min. design metal temp.  	-20	 ºF at 	655	 psi.  Hydro., pneu., or comb. Test pressure  	HYDRO AT	852	  psi.

Proof Test   _____--____

10.  Nozzles, inspection and safety valve openings:

| Purpose (Inlet, Outlet, Drain) | No. | Diameter or Size | Type | Material |  | Nozzle Thickness |  | Reinforcement Matl. | Attachment Details |  | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Nozzle | Flange | Nom. | Corr. |  | Nozzle | Flange |  |
| INLET/OUTLET/ INSP. | 2 | 2” | RFWN CL.300 | SA-106-B | SA-105-N | 0.344” | 0.0625” | - | UW-16.1(c) | TYPE1 | HEAD/ SHELL |
| DRAIN/LC | 2 | 2” | TOL | - | SA-105-N | CL.6000 | 0.0625” | - | UW-16.1(a) | WELDED | HEAD/ SHELL |
| LSHH | 1 | 1 1/2" | TOL | - | SA-105-N | CL.6000 | 0.0625” | - | UW-16.1(a) | WELDED | SHELL |
| DUMP/INSP./SPARE/ EQUALIZATION | 3 | 1” | TOL | - | SA-105-N | CL.6000 | 0.0625” | - | UW-16.1(a) | WELDED | SHELL |
| LG/TI | 3 | 3/4" | TOL | - | SA-105-N | CL.6000 | 0.0625” | - | UW-16.1(a) | WELDED | SHELL |

11.  Supports: Skirt  	NO	 Lugs  	NA	 Legs  	3	 Other  	N/A	 Attached  	SHELL/WELDED

(Yes or No)	(No.)	(No.)	(Describe)	(Where and how)

12.  Remarks: Manufacturer's Partial Data Reports properly identified and signed by Commissioned Inspectors have been furnished for the following items of this report:

NONE

(Name of part, item number. Mfgr's name and identifying stamp)

SERVICE: SWEET                                                                                      	                                    PSV ON PIPE AS PER UG-125

CONST. DRAWING NO.: 10569B-211 REV.1                                                                                                                                  RADIOGRAPHY:FULL UW-11(a)                   TESTING: NO CHARPY IMPACT TEST REQUIRED PER UG-20(f), (1-5)		    CUBIC CAPACITY: 4.28 CUBIC FEET

# CERTIFICATE OF SHOP/FIELD COMPLIANCE We certify that the statements made in this report are correct and that all details of design, material, construction, and workmanship of this vessel conform to the ASME Code for Pressure Vessels, Section VIII, Division 1. "U" Certificate of Authorization No. 28071 expires December 7 , 2015 . Date Co. name Enerflex Ltd, Signed (Manufacturer) (Representative)

# CERTIFICATE OF SHOP/FIELD INSPECTION Vessel constructed by Enerflex Ltd, at CALGARY, ALBERTA, CANADA . I, the undersigned, holding a valid commission issued by The National Board of Boiler and Pressure Vessel Inspectors and/or the State or Province of ALBERTA and employed by ABSA have inspected the component described in this Manufacturer's Data Report on , 20 , and state that, to the best of my knowledge and belief, the Manufacturer has constructed this pressure vessel in accordance with ASME Code, Section VIII, Division 1. By signing this certificate neither the Inspector nor his/her employer makes any warranty, expressed or implied, concerning the pressure vessel described in this Manufacturer's Data Report. Furthermore, neither the Inspector nor his/her employer shall be liable in any manner for any personal injury or property damage or a loss of any kind arising from or connected with this inspection. Date Signed Commissions ALBERTA (Authorized Inspector) (Nat'l Board. (incl. endorsements) State, Prov. and No.)