|  | OPERATING MANUAL UNIT COMPONENT DATA | UNIT # 051009 MODEL: B200 Page: 1 of 2 |
| --- | --- | --- |

FOR:  ELL/ Winter Petroleum             AREA:   Rainbow Lake             LSD:  10-24-110-04W6M

ENGINE  	CATERPILLAR G3306TA      S/N: G 6X01555

Ignition:	Mag:  Altronic III 6A29H-A0	Coils: Altronic	P/N: 591011B

Plugs:    Champion RTN79G                     	Power Source:   N/A

Starter:   TDI, Std T306-60015-02R

Oil Filter: Cat P/N IR0716

Air Filter:  6N6444 (Inner)   7W5313 (Outer)    Fuel Filter: Parker 200-35-DX Element

CRN# : OE 7415.5

COOLER	AIR-X-CHANGERS 72AVI

Fan:                      Moore 72”, 4 Blade, Left Hand, CL5K	Pitch:  6.40 Angle at Clevis

Shaft Bearings: 	Main:  2-3/16” Dodge S2000     	Idler:  2-3/16 Dodge FB/CC203

Drive Belts:  	BX-120 (2)	Louvre Actuator:  Garzo 501-12

Coolant Hoses:     Engine In:  NAPA 7231 2” straight           Engine Out:  NAPA 7197 1-3/4” 900

AW Sect. S/N: 055580        JW Sect. S/N: 055580

IC   Sect. S/N: 055580.3     MAWP: 1292       AC Sect S/N: 055580.4       MAWP: 1292

CRN# : R8395.213                                         CRN#  R7796.213

PROCESS	Filter Coalescer:  16”   " Dia     S/N: 050625-01      MAWP: 707 Psig

CRN# : P 8055.213 @ 545079

Elements:  (         )  PECO FG-36-MS

Relief Valves: LHS (Cyl #A2)                Farris ‘D’   set @      207      Psig

RHS (Cyl #2)                                         Farris ‘C’   set @     1292    Psig

Orifices:		Refer to the Process Flow Sheet in the Drawing Section of your Operating Manual.

* Auto Bypass:  Model:  Fisher D-4        Orifice:  ½”           Action:  F/O

Compressor Unit Inlet System:   Refer to Process Flow Sheet in the Drawing section of your

Operating Manual.

Checked By:

|  | OPERATING MANUAL UNIT COMPONENT DATA | UNIT # 051009 MODEL: B200 Page: 2 of 2 |
| --- | --- | --- |

PANEL	Process Pressure Gauge Ranges

CYL #1 suction      	: 	0 - 300     Psig

CYL #1 discharge	:  	0 - 1000   Psig

CYL #2 discharge   	:	0 - 1500   Psig

Note:  Process Pressure and Temperature Shutdown Settings should be reset to reflect actual

operating field conditions.

FLUIDS	Oil:  Petro Canada Sentinal     445      Engine Capacity:  151 litres (33.5 Imp gal)

Comp. Capacity:  15 litres (3.3 Imp gal)

Engine Coolant:  UCARTHERM  50/50  Capacity:  130 litres (29 Imp gal)

COMPRESSOR

ARIEL  JGA/2  Frame S/N:   F 21555

Oil Filter:  Ariel P/N:  A-0661

CONFIGURATION                        THROW #2            THROW #1

Cylinder # 	                 A2                       A1

Stage #                                                 1                         2

Cylinder Bore, In.              	                 4 ¾”                    3”

Class                                                    JG   	           JG

MAWP, Psig   	                 865	           2550

Cylinder S/N            	                 C-71172             C- 65737

CLEARANCE FEATURES

High Clearance Assy., Qty:                  N/R	              N/R

VVCP, S/N (or NA):		   U-43989               U-40333

VALVES

Suction:	Part # (as shipped)            B-3395-S             B-2930-CC

Discharge:	Part # (as shipped)            B-3396-DD          B-2929-P

Valve Operating Ranges:  Refer to the Performance Section in your Operating Manual.

BALANCE AND LUBRICATION:	Refer to the Frame section in your Operating Manual.

COUPLING

Type: 		350CMR

ELECTRICAL 	CSA Reg#:

* Denotes Options

Checked by: