| Facility: |  | Chinchaga C-32-H/94-H-8 |  |  |  |  |  |  |  |  | PRN: |  |  |  | PV69P8797 (BC) |  |  |  |  | Date of Inspection: |  |  |  |  |  |  | 8-Jun-2017 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equipment Type: |  |  | Separator (BC #1) |  |  |  |  |  |  |  |  |  |  |  |  | Field Tag #: |  |  |  |  |  |  | 603VS008 |  |  |  |  |  |
| Service: | ☐Sour ☒Sweet ☐Cryogenic ☐Air ☐Steam |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MAWP: |  |  | 275 PSI @120F |  |  |  |  |  |  |  |  |  |
| Fluid: | Emulsion |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Manway |  |  | ☒ Yes |  |  |  |  | ☐ No |  | Size of Manway |  |  |
| Scope of Inspection: |  |  |  |  | Type of Inspection: |  |  |  |  |  |  |  |  |  |  |  |  | Cleaning: |  |  |  |  |  |  |  | Access: |  |  |
| ☐ Pre-commissioning |  |  |  |  | ☐ AE |  |  |  | ☐ MT |  |  |  | ☐ RT |  |  |  |  | ☒ Good |  |  |  |  |  |  |  | ☒ Full |  |  |
| ☐ Initial |  |  |  |  | ☐ ET |  |  |  | ☐MFL |  |  |  | ☐ UT |  |  |  |  | ☐ Fair |  |  |  |  |  |  |  | ☐ Partial |  |  |
| ☒ Re-inspection |  |  |  |  | ☐ LT |  |  |  | ☐PT |  |  |  | ☒ VT |  |  |  |  | ☐ Inadequate |  |  |  |  |  |  |  | ☐ External |  |  |
|  |  |  |  |  | ☐HT |  |  |  | ☐Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: Add pages and sketches as needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  | ☐ External |  |  |  |  |  |  |  |  |  | ☒Internal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal: - Manway free of pits - Vessel is uncoated - Manway shell is in excellent condition. No pitting observed. - 1st stage of shell is in excellent condition circumferentially. Some general erosion corrosion on splash impact area from inlet diverter. - 2nd stage (past weir) nozzles in good condition, no visible cracking or pitting. - Weir in good condition - Drain nozzle in good condition. - No general corrosion observed, oily residue persistent throughout vessel surfaces. - Glycol heat lines have stainless fittings and are in like new condition. - Glycol heat trace lines are in fouled externally and unable to inspect. Small portion scraped and inspected has no pitting corrosion observed. - Bottom end of shell is in good condition with no pitting observed. Nozzles in good condition, no pits or cracks. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| External: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Recommendations: Perform UT to determine remaining wall on vessel, particularly on splash area from inlet diverter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suitable for further service: |  |  |  | ☒Yes |  |  | ☐No |  |  | Remedial Action Priority Level: |  |  |  |  |  |  |  |  |  |  |  | ☐High |  |  | ☐Medium |  |  | ☐Low |
| Inspection Performed By: |  |  |  | Darren Isaac |  |  |  |  |  |  |  |  |  | Click here to enter text. |  |  |  |  |  |  | Date of Report: |  |  |  |  | 12-Jun-2017 |  |  |
| Signature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Follow Up Inspection Required: |  |  |  |  |  | ☐ Yes |  | ☒ No |  |  |  | By Date: |  |  |  |  | DD-Mon-YYYY |  |  |  |  |  |  |  |  |  |  |  |

Pictures: Figure 1 Manway Figure 2 Internal Overview Figure 3 Weir Figure 4 Nozzles Figure 5 PSV Nozzle Figure 6 Upper Shell Nozzle Figure 7 Lower Shell Drain Nozzle Figure 8 Glycol Heat Trace Lines and Boot Figure 9 South Floor Drain Nozzle Figure 10 Looking Down Into The Boot Figure 11 Lower Shell on Boot Figure 12 Glycol Heat Trace Nozzle and Piping Figure 13 Glycol Heat Trace Nozzle and Piping Top View Figure 14 Inlet and Diverter Plate Figure 15 Nozzle Top Down view